P17 Problem 1.1

- Show that the NOR gate is universal.

- Show that the 2-to-1 multiplexer is universal.

- Is there any other two-input element, besides NAND and NOR, that is universal?

Answer to d. Show how to build AND, OR, and NOT gate from NOR gate.

Answer to e. Show how to build AND, OR, and NOT gate from 2-to-1 multiplexer.

F=BA+B=BA+B+A=A(1+B)+B=A+B

Answer to f.  Yes,

P17 Problem 1.5 BCD-to-7segment decoder

- Inputs are BCD digits; the digits 6, 7, and 9 are displayed with 6, 3, and 6 segments.

Answer:

| Input | Output |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| x3x2x1x0 | e6 | e5 | e4 | e3 | e2 | e1 | e0 |
| 0000 | 0 | 1 | 1 | 1 | 1 | 1 | 1 |
| 0001 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |
| 0010 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0011 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| 0100 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |
| 0101 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 0110 | 1 | 1 | 1 | 1 | 1 | 0 | 1 |
| 0111 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| 1000 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1001 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |

e6      x1x0

|  |  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| x3x2 | 00 | 0 | 0 | 1 | 1 |
|  | 01 | 1 | 1 | 0 | 1 |
|  | 11 | x | x | x | x |
|  | 10 | 1 | 1 | x | x |

e6=x3x1+x2x1+x2x1+x1x0

e5      x1x0

|  |  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| x3x2 | 00 | 1 | 0 | 0 | 0 |
|  | 01 | 1 | 1 | 0 | 1 |
|  | 11 | x | x | x | x |
|  | 10 | 1 | 1 | x | x |

e5=x3x1+x2x1+x2x0+x1x0

e4      x1x0

|  |  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| x3x2 | 00 | 1 | 0 | 0 | 1 |
|  | 01 | 0 | 0 | 0 | 1 |
|  | 11 | x | x | x | x |
|  | 10 | 1 | 0 | x | x |

e4=x2x0+x1x0

e3      x1x0

|  |  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| x3x2 | 00 | 1 | 0 | 1 | 1 |
|  | 01 | 0 | 1 | 0 | 1 |
|  | 11 | x | x | x | x |
|  | 10 | 1 | 1 | x | x |

e3=x3+x2x0+x2x1+x1x0+x2x1x0

e2      x1x0

|  |  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| x3x2 | 00 | 1 | 1 | 1 | 0 |
|  | 01 | 1 | 1 | 1 | 1 |
|  | 11 | x | x | x | x |
|  | 10 | 1 | 1 | x | x |

e2=x2+x1+x0

e1      x1x0

|  |  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| x3x2 | 00 | 1 | 1 | 1 | 1 |
|  | 01 | 1 | 0 | 1 | 0 |
|  | 11 | x | x | x | x |
|  | 10 | 1 | 1 | x | x |

e1=x3+x3x2+x1x0+x1x0

e0      x1x0

|  |  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- |
| x3x2 | 00 | 1 | 0 | 1 | 1 |
|  | 01 | 0 | 1 | 1 | 1 |
|  | 11 | x | x | x | x |
|  | 10 | 1 | 1 | x | x |

e0=x3+x1+x2x0+x2x0

Circuit diagram:

e2

e6

e1

x3x2x1x0

e5

e0

e4

e3

P18  Problem 1.10

Prove that the following pairs of logic expressions are equivalent, first by truth table, and then by means of symbolic manipulation.

Proof:

RHS=xyz+xyz+xyz+xyz

=xyz+xyz+xyz+xyz

=(xy+xy)z+(xy+xy)z

=(x⊕y)z+(x⊕y)z

=x⊕y⊕z=LHS

Truth table:

| Input xyz | LHS x⊕y⊕z | RHS xyz+xyz+xyz+xyz |
| --- | --- | --- |
| 000 | 0 | 0 |
| 001 | 1 | 1 |
| 010 | 1 | 1 |
| 011 | 0 | 0 |
| 100 | 1 | 1 |
| 101 | 0 | 0 |
| 110 | 0 | 0 |
| 111 | 1 | 1 |

P18  Problem 1.10 d     Proof:

LHS=xz+wyz+wx {y+wxy+xyz

=xz+wyz+wx {y(z+z)+wxy(z+z)+(w+w)xyz

=xz+wyz+wx {yz+wx {yz+wxyz+wxy {z+wxyz+wxyz

=xz+wyz+wx {yz+wx {yz+wxyz+yz(wx+wx+wx)

=xz+wyz+wx {yz+wx {yz+wxyz+yz(w(x+x)+wx)

=xz+wyz+wx {yz+wx {yz+wxyz+wxyz+wyz

=xz+wx {yz+wxyz+wyz+wx {yz+wxyz+wyz

=xz(1+wy+wy)+wyz+wx {yz+wxyz+wyz

=xz+wyz+wyz+wx {yz+wxyz

=xz+wyz+wx {y+wyz+wx {yz+wxyz

=xz+wyz+wx {y(1+z)+wyz+wxyz

=xz+wyz+wx {y+wyz+wxyz

=xz+wyz+wyz+wxyz

=xz+wyz(1+x)+wyz+wxyz

=xz+wyz+wyzx+wyz+wxyz

=xz+wyz+wxz(y+y)+wyz

=xz+wyz+wyz+wxz=RHS

Truth table:

| Input wxyz | LHS | RHS |
| --- | --- | --- |
| 0000 | 0 | 0 |
| 0001 | 0 | 0 |
| 0010 | 1 | 1 |
| 0011 | 0 | 0 |
| 0100 | 0 | 0 |
| 0101 | 1 | 1 |
| 0110 | 1 | 1 |
| 0111 | 1 | 1 |
| 1000 | 1 | 1 |
| 1001 | 0 | 0 |
| 1010 | 1 | 1 |
| 1011 | 0 | 0 |
| 1100 | 1 | 1 |
| 1101 | 1 | 1 |
| 1110 | 0 | 0 |
| 1111 | 1 | 1 |

p.18  Problem 1.12

Assume x = (x2x1x0) two and (y2y1y0) two are 3-bit unsigned binary numbers. Write down a logic expression in terms of the six Booleans variables x2, x1, x0, y2, y1, and  y0 that assumes the value 1 iff:

- x = y

- x < y

- x ≤ y

Answer to a:

f1=x2⊕y2⋅x1⊕y1⋅x0⊕y0

Answer to b:

f2=y2x2+x2⊕y2⋅y1x1+x2⊕y2⋅x1⊕y1⋅y0x0

Answer to c:

f3=f1+f2x2⊕y2⋅x1⊕y1⋅x0⊕y0+y2x2+x2⊕y2⋅y1x1+x2⊕y2⋅x1⊕y1⋅y0x0